# MSK Modem Registers

Do not edit. This document is generated from the following input documents:

src/regblock_udps.rdl

src/msk_top_regs.rdl

Top level node: msk_top_regs

# msk_top_regs address map

Absolute Address: 0x0

Base Offset: 0x0

Size: 0xAC

MSK Modem Configuration and Status Registers

| Offset | Identifier | Name |
| --- | --- | --- |
| 0x00 | Hash_ID_Low | Pluto MSK FPGA Hash ID - Lower 32-bits |
| 0x04 | Hash_ID_High | Pluto MSK FPGA Hash ID - Upper 32-bits |
| 0x08 | MSK_Init | MSK Modem Initialization Control |
| 0x0C | MSK_Control | MSK Modem Control |
| 0x10 | MSK_Status | MSK Modem Status 0 |
| 0x14 | Tx_Bit_Count | MSK Modem Status 1 |
| 0x18 | Tx_Enable_Count | MSK Modem Status 2 |
| 0x1C | Fb_FreqWord | Bitrate NCO Frequency Control Word |
| 0x20 | TX_F1_FreqWord | Tx F1 NCO Frequency Control Word |
| 0x24 | TX_F2_FreqWord | Tx F2 NCO Frequency Control Word |
| 0x28 | RX_F1_FreqWord | Rx F1 NCO Frequency Control Word |
| 0x2C | RX_F2_FreqWord | Rx F2 NCO Frequency Control Word |
| 0x30 | LPF_Config_0 | PI Controller Configuration and Low-pass Filter Configuration |
| 0x34 | LPF_Config_1 | PI Controller Configuration Configuration Register 1 |
| 0x38 | Tx_Data_Width | Modem Tx Input Data Width |
| 0x3C | Rx_Data_Width | Modem Rx Output Data Width |
| 0x40 | PRBS_Control | PRBS Control 0 |
| 0x44 | PRBS_Initial_State | PRBS Control 1 |
| 0x48 | PRBS_Polynomial | PRBS Control 2 |
| 0x4C | PRBS_Error_Mask | PRBS Control 3 |
| 0x50 | PRBS_Bit_Count | PRBS Status 0 |
| 0x54 | PRBS_Error_Count | PRBS Status 1 |
| 0x58 | LPF_Accum_F1 | F1 PI Controller Accumulator |
| 0x5C | LPF_Accum_F2 | F2 PI Controller Accumulator |
| 0x60 | axis_xfer_count | MSK Modem Status 3 |
| 0x64 | Rx_Sample_Discard | Rx Sample Discard |
| 0x68 | LPF_Config_2 | PI Controller Configuration Configuration Register 2 |
| 0x6C | f1_nco_adjust | F1 NCO Frequency Adjust |
| 0x70 | f2_nco_adjust | F2 NCO Frequency Adjust |
| 0x74 | f1_error | F1 Error Value |
| 0x78 | f2_error | F2 Error Value |
| 0x7C | Tx_Sync_Ctrl | Transmitter Sync Control |
| 0x80 | Tx_Sync_Cnt | Transmitter Sync Duration |
| 0x84 | Tx_Sync_Pat | Transmitter Sync pattern |
| 0x88 | lowpass_ema_alpha1 | Exponential Moving Average Alpha |
| 0x8C | lowpass_ema_alpha2 | Exponential Moving Average Alpha |
| 0x90 | rx_power | Receive Power |
| 0x94 | tx_async_fifo_rd_wr_ptr | Tx async FIFO read and write pointers |
| 0x98 | rx_async_fifo_rd_wr_ptr | Rx async FIFO read and write pointers |
| 0x9C | rx_frame_sync_status | Frame Sync Status |
| 0xA0 | symbol_lock_control | Symbol Lock Control |
| 0xA4 | symbol_lock_status | Symbol Lock Status |
| 0xA8 | symbol_lock_time | Symbol Lock Time |

## Hash_ID_Low register

Absolute Address: 0x0

Base Offset: 0x0

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | hash_id_lo | r | 0xAAAA5555 | Hash ID Lower 32-bits |

### hash_id_lo field

Lower 32-bits of Pluto MSK FPGA Hash ID

## Hash_ID_High register

Absolute Address: 0x4

Base Offset: 0x4

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | hash_id_hi | r | 0x5555AAAA | Hash ID Upper 32-bits |

### hash_id_hi field

Upper 32-bits of Pluto MSK FPGA Hash ID

## MSK_Init register

Absolute Address: 0x8

Base Offset: 0x8

Size: 0x4

Synchronous initialization of MSK Modem functions, does not affect configuration registers.

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | txrxinit | rw | 0x1 | Tx/Rx Init Enable |
| 1 | txinit | rw | 0x1 | Tx Init Enable |
| 2 | rxinit | rw | 0x1 | Rx Init Enable |

### txrxinit field

0 -> Normal modem operation

### txinit field

0 -> Normal Tx operation

### rxinit field

0 -> Normal Rx operation

## MSK_Control register

Absolute Address: 0xC

Base Offset: 0xC

Size: 0x4

MSK Modem Configuration and Control

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | ptt | rw | 0x0 | Push-to-Talk Enable |
| 1 | loopback_ena | rw | 0x0 | Modem Digital Tx -> Rx Loopback Enable |
| 2 | rx_invert | rw | 0x0 | Rx Data Invert Enable |
| 3 | clear_counts | rw | 0x0 | Clear Status Counters |
| 4 | diff_encoder_loopback | rw | 0x0 | Differential Encoder -> Decoder Loopback Enable |
| 7:5 | reserved | rw | 0x0 | — |
| 10:8 | tx_shift | rw | 0x0 | Tx Sample Shoift |

### ptt field

0 -> PTT Disabled

### loopback_ena field

0 -> Modem loopback disabled

### rx_invert field

0 -> Rx data normal

### clear_counts field

Clear Tx Bit Counter and Tx Enable Counter

### diff_encoder_loopback field

0 -> Differential Encoder -> Decoder loopback disabled

### reserved field

Reserved

### tx_shift field

Shift left from 0 to 4 bits

## MSK_Status register

Absolute Address: 0x10

Base Offset: 0x10

Size: 0x4

Modem status bits

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | demod_sync_lock | r | 0x0 | Demodulator Sync Status |
| 1 | tx_enable | r | 0x0 | AD9363 DAC Interface Tx Enable Input Active |
| 2 | rx_enable | r | 0x0 | AD9363 ADC Interface Rx Enable Input Active |
| 3 | tx_axis_valid | r | 0x0 | Tx S_AXIS_VALID |

### demod_sync_lock field

Demodulator Sync Status - not currently implemented

### tx_enable field

1 -> Data to DAC Enabled

### rx_enable field

1 -> Data from ADC Enabled

### tx_axis_valid field

1 -> S_AXIS_VALID Enabled

## Tx_Bit_Count register

Absolute Address: 0x14

Base Offset: 0x14

Size: 0x4

Modem status data

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | Tx Bit Count |

### data field

Count of data requests made by modem

## Tx_Enable_Count register

Absolute Address: 0x18

Base Offset: 0x18

Size: 0x4

Modem status data

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | Tx Enable Count |

### data field

Number of clocks on which Tx Enable is active

## Fb_FreqWord register

Absolute Address: 0x1C

Base Offset: 0x1C

Size: 0x4

Set Modem Data Rate

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | Frequency Control Word |

### config_data field

Sets the center frequency of the NCO as FW = Fn * 2^32/Fs, where Fn is the desired NCO frequency, and Fs is the NCO sample rate

## TX_F1_FreqWord register

Absolute Address: 0x20

Base Offset: 0x20

Size: 0x4

Set Modulator F1 Frequency

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | Frequency Control Word |

### config_data field

Sets the center frequency of the NCO as FW = Fn * 2^32/Fs, where Fn is the desired NCO frequency, and Fs is the NCO sample rate

## TX_F2_FreqWord register

Absolute Address: 0x24

Base Offset: 0x24

Size: 0x4

Set Modulator F2 Frequency

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | Frequency Control Word |

### config_data field

Sets the center frequency of the NCO as FW = Fn * 2^32/Fs, where Fn is the desired NCO frequency, and Fs is the NCO sample rate

## RX_F1_FreqWord register

Absolute Address: 0x28

Base Offset: 0x28

Size: 0x4

Set Demodulator F1 Frequency

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | Frequency Control Word |

### config_data field

Sets the center frequency of the NCO as FW = Fn * 2^32/Fs, where Fn is the desired NCO frequency, and Fs is the NCO sample rate

## RX_F2_FreqWord register

Absolute Address: 0x2C

Base Offset: 0x2C

Size: 0x4

Set Demodulator F2 Frequency

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | Frequency Control Word |

### config_data field

Sets the center frequency of the NCO as FW = Fn * 2^32/Fs, where Fn is the desired NCO frequency, and Fs is the NCO sample rate

## LPF_Config_0 register

Absolute Address: 0x30

Base Offset: 0x30

Size: 0x4

Configure PI controller and low-pass filter

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | lpf_freeze | rw | 0x0 | Freeze the accumulator's current value |
| 1 | lpf_zero | rw | 0x0 | Hold the PI Accumulator at zero |
| 7:2 | prbs_reserved | rw | 0x0 | Reserved |
| 31:8 | lpf_alpha | rw | 0x0 | Lowpass IIR filter alpha |

### lpf_freeze field

0 -> Normal operation

### lpf_zero field

0 -> Normal operation

### lpf_alpha field

Value controls the filter rolloff

## LPF_Config_1 register

Absolute Address: 0x34

Base Offset: 0x34

Size: 0x4

Configures PI Controller I-gain and divisor

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 23:0 | i_gain | rw | 0x0 | Integral Gain Value |
| 31:24 | i_shift | rw | 0x0 | Integral Gain Bit Shift |

### i_gain field

Value m of 0-16,777,215 sets the integral multiplier

### i_shift field

Value n of 0-32 sets the integral divisor as 2^-n

## Tx_Data_Width register

Absolute Address: 0x38

Base Offset: 0x38

Size: 0x4

Set the parallel data width of the parallel-to-serial converter

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 7:0 | data_width | rw | 0x8 | Modem input/output data width |

### data_width field

Set the data width of the modem input/output

## Rx_Data_Width register

Absolute Address: 0x3C

Base Offset: 0x3C

Size: 0x4

Set the parallel data width of the serial-to-parallel converter

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 7:0 | data_width | rw | 0x8 | Modem input/output data width |

### data_width field

Set the data width of the modem input/output

## PRBS_Control register

Absolute Address: 0x40

Base Offset: 0x40

Size: 0x4

Configures operation of the PRBS Generator and Monitor

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | prbs_sel | rw | 0x0 | PRBS Data Select |
| 1 | prbs_error_insert | w | 0x0 | PRBS Error Insert |
| 2 | prbs_clear | w | 0x0 | PRBS Clear Counters |
| 3 | prbs_manual_sync | w | 0x0 | PRBS Manual Sync |
| 15:4 | prbs_reserved | rw | 0x0 | Reserved |
| 31:16 | prbs_sync_threshold | rw | 0x0 | PRBS Auto Sync Threshold |

### prbs_sel field

0 -> Select Normal Tx Data 1 -> Select PRBS Tx Data

### prbs_error_insert field

0 -> 1 : Insert bit error in Tx data (both Normal and PRBS)

### prbs_clear field

0 -> 1 : Clear PRBS Counters

### prbs_manual_sync field

0 -> 1 : Synchronize PRBS monitor

### prbs_sync_threshold field

0 : Auto Sync Disabled

## PRBS_Initial_State register

Absolute Address: 0x44

Base Offset: 0x44

Size: 0x4

PRBS Initial State

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | PRBS Seed |

### config_data field

Sets the starting value of the PRBS generator

## PRBS_Polynomial register

Absolute Address: 0x48

Base Offset: 0x48

Size: 0x4

PRBS Polynomial

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | PRBS Polynomial |

### config_data field

Bit positions set to '1' indicate polynomial feedback positions

## PRBS_Error_Mask register

Absolute Address: 0x4C

Base Offset: 0x4C

Size: 0x4

PRBS Error Mask

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | config_data | rw | 0x0 | PRBS Error Mask |

### config_data field

Bit positions set to '1' indicate bits that are inverted when a bit error is inserted

## PRBS_Bit_Count register

Absolute Address: 0x50

Base Offset: 0x50

Size: 0x4

PRBS Bits Received

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | PRBS Bits Received |

### data field

Number of bits received by the PRBS monitor since last BER can be calculated as the ratio of received bits to errored-bits

## PRBS_Error_Count register

Absolute Address: 0x54

Base Offset: 0x54

Size: 0x4

PRBS Bit Errors

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | PRBS Bit Errors |

### data field

Number of errored-bits received by the PRBS monitor since last sync BER can be calculated as the ratio of received bits to errored-bits

## LPF_Accum_F1 register

Absolute Address: 0x58

Base Offset: 0x58

Size: 0x4

Value of the F1 PI Controller Accumulator

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | PI Controller Accumulator Value |

### data field

PI Controller Accumulator Value

## LPF_Accum_F2 register

Absolute Address: 0x5C

Base Offset: 0x5C

Size: 0x4

Value of the F2 PI Controller Accumulator

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | PI Controller Accumulator Value |

### data field

PI Controller Accumulator Value

## axis_xfer_count register

Absolute Address: 0x60

Base Offset: 0x60

Size: 0x4

Modem status data

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | S_AXIS Transfers |

### data field

Number completed S_AXIS transfers

## Rx_Sample_Discard register

Absolute Address: 0x64

Base Offset: 0x64

Size: 0x4

Configure samples discard operation for demodulator

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 7:0 | rx_sample_discard | rw | 0x0 | Rx Sample Discard Value |
| 15:8 | rx_nco_discard | rw | 0x0 | Rx NCO Sample Discard Value |

### rx_sample_discard field

Number of Rx samples to discard

### rx_nco_discard field

Number of NCO samples to discard

## LPF_Config_2 register

Absolute Address: 0x68

Base Offset: 0x68

Size: 0x4

Configures PI Controller I-gain and divisor

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 23:0 | p_gain | rw | 0x0 | Proportional Gain Value |
| 31:24 | p_shift | rw | 0x0 | Proportional Gain Bit Shift |

### p_gain field

Value m of 0-16,777,215 sets the proportional multiplier

### p_shift field

Value n of 0-32 sets the proportional divisor as 2^-n

## f1_nco_adjust register

Absolute Address: 0x6C

Base Offset: 0x6C

Size: 0x4

Status Register

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Frequency offet applied to the F1 NCO

## f2_nco_adjust register

Absolute Address: 0x70

Base Offset: 0x70

Size: 0x4

Status Register

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Frequency offet applied to the F2 NCO

## f1_error register

Absolute Address: 0x74

Base Offset: 0x74

Size: 0x4

Status Register

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Error value of the F1 Costas loop after each active bit period

## f2_error register

Absolute Address: 0x78

Base Offset: 0x78

Size: 0x4

Status Register

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Error value of the F2 Costas loop after each active bit period

## Tx_Sync_Ctrl register

Absolute Address: 0x7C

Base Offset: 0x7C

Size: 0x4

Provides control bits for generation of transmitter synchronization patterns

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | tx_sync_ena | rw | 0x0 | Tx Sync Enable |
| 1 | tx_sync_force | rw | 0x0 | Tx Sync Force |

### tx_sync_ena field

0 : Disable sync transmission

### tx_sync_force field

0 : Normal operation

## Tx_Sync_Cnt register

Absolute Address: 0x80

Base Offset: 0x80

Size: 0x4

Sets the duration of the synchronization tones when enabled

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 23:0 | tx_sync_cnt | rw | 0x0 | Tx sync duration |

### tx_sync_cnt field

Value from 0x00_0000 to 0xFF_FFFF.

## Tx_Sync_Pat register

Absolute Address: 0x84

Base Offset: 0x84

Size: 0x4

Sets the synchronization pattern to be transmitted when synchronization tones are enabled

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 15:0 | tx_sync_pat | rw | 0x1B33 | Tx sync pattern |

### tx_sync_pat field

Value from 0x0000 to 0xFFFF.

## lowpass_ema_alpha1 register

Absolute Address: 0x88

Base Offset: 0x88

Size: 0x4

Sets the alpha for the EMA

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 17:0 | alpha | rw | 0x0 | EMA alpha |

### alpha field

Value from 0x0_0000 to 0x3_FFFF represent the EMA alpha

## lowpass_ema_alpha2 register

Absolute Address: 0x8C

Base Offset: 0x8C

Size: 0x4

Sets the alpha for the EMA

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 17:0 | alpha | rw | 0x0 | EMA alpha |

### alpha field

Value from 0x0_0000 to 0x3_FFFF represent the EMA alpha

## rx_power register

Absolute Address: 0x90

Base Offset: 0x90

Size: 0x4

Receive power computed from I/Q samples

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 22:0 | data | rw | 0x0 | Receive Power |

### data field

Value that represent the RMS power of the incoming signal (I-channel)

## tx_async_fifo_rd_wr_ptr register

Absolute Address: 0x94

Base Offset: 0x94

Size: 0x4

Tx async FIFO read and write pointers

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Read and Write Pointers

## rx_async_fifo_rd_wr_ptr register

Absolute Address: 0x98

Base Offset: 0x98

Size: 0x4

Rx async FIFO read and write pointers

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | — |

### data field

Read and Write Pointers

## rx_frame_sync_status register

Absolute Address: 0x9C

Base Offset: 0x9C

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | frame_sync_locked | r | 0x0 | Frame Sync Lock |
| 1 | frame_buffer_overflow | r | 0x0 | Frame Buffer Overflow |
| 25:2 | frames_received | rw | 0x0 | Frames Received |
| 31:26 | frame_sync_errors | rw | 0x0 | Frames Sync Errors |

### frame_sync_locked field

0 - Frame sync not locked 1 - Frame sync locked

### frame_buffer_overflow field

0 - Normal operation 1 - Buffer overflow

### frames_received field

Count of frames received. Value is 0x00_0000 to 0xFF_FFFF. Counter rolls over when max count is reached.

### frame_sync_errors field

Count of frame sync errors. Value is 0 to 63. Counter rolls over when max count is reached.

## symbol_lock_control register

Absolute Address: 0xA0

Base Offset: 0xA0

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 9:0 | symbol_lock_count | rw | 0x80 | Symbol Lock Integration Count |
| 25:10 | symbol_lock_threshold | rw | 0x2710 | Symbol Lock Threshold |

### symbol_lock_count field

Sets the integration period in symbols. Value is from 0 to 1023.

### symbol_lock_threshold field

Sets the threshold value on which to declare symbol sync

## symbol_lock_status register

Absolute Address: 0xA4

Base Offset: 0xA4

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 0 | f1f2 | r | 0x0 | Symbol Lock F1 and F2 |
| 1 | f1 | r | 0x0 | Symbol Lock F1 |
| 2 | f2 | r | 0x0 | Symbol Lock F2 |
| 3 | unlock_f1 | r | 0x0 | F1 unlocked since last read |
| 4 | unlock_f2 | r | 0x0 | F2 unlocked since last read |

### f1f2 field

0 - Unlocked 1 - F1 and F2 locked

### f1 field

0 - Unlocked 1 - F1 locked

### f2 field

0 - Unlocked 1 - F2 locked

### unlock_f1 field

0 - No unlock since last read 1 - One or mode unlocks since last read

### unlock_f2 field

0 - No unlock since last read 1 - One or mode unlocks since last read

## symbol_lock_time register

Absolute Address: 0xA8

Base Offset: 0xA8

Size: 0x4

| Bits | Identifier | Access | Reset | Name |
| --- | --- | --- | --- | --- |
| 15:0 | f1 | r | 0x0 | F1 Symbol Lock Time |
| 31:16 | f2 | r | 0x0 | F2 Symbol Lock Time |

### f1 field

Number of symbols for F1 lock since init released

### f2 field

Number of symbols for F2 lock since init released